# INTEGRATED PATIO COVER WITH ROTATABLE AND RETRACTABLE ALUMINIUM LOUVRES

## Manufacturer

RENSON Outdoor nv, Polydore Rensonstraat 8, 9770 Kruisem – Belgium
Tel. +32(0)56 30 30 00, info@renson.be, www.renson-outdoor.com

(You can omit the text highlighted in red according to your choice)

## Description

## Aero Skye® is a horizontal patio cover integrated into an existing structure and equipped with a system of aluminium extruded, rotatable and retractable louvres.

## Dimensions

Span:	Min. 1800 mm
	Max. 4500 mm

Pivot:	Min. 3250 mm
	Max. 6200 mm

Free passage height: 	Dependent on the supporting structure

For your information:
The span side corresponds to the span of the blades.
The pivot side is the side on which the blades rotate (pivot)

## System implementation

### Frame:

- Aluminium-extruded profiles, powder-coated in a colour of your choice

- Connections on the same side are finished in a single plane, without protruding cover caps or visible screws

- Height: 260 mm

- Height with louvres in vertical position: 260 mm + 95 mm

- Depth of span-frame profile: 150 mm

- Depth of pivot-frame profile: 330 mm

- Inclination: 0°

### Blades:

- Sun protection blades made of extruded aluminium

- Double-walled for handling a heavy snow load

  - 135° rotatable (electronically controlled)

  - Slope: 3 cm over the full louvre length for optimal water drainage

  - Underside completely flat

  - Sides fitted with an L-profile to prevent backflowing water, welded and coated in the same colour as the louvres

  - A triple-folded edge along the louvre length prevents residual water from dripping onto the terrace and furniture when the roof opens

  - Secured on the pivot sides using stainless-steel shafts with integrated locking for a stable construction

### Motorisation:

- 4 in-line 24 V DC motors with planetary transmission

- How it works

  - Rotation: Worm and worm gear

  - Retraction/opening: Synchronised spindles with a trolley and chain system
Even and guaranteed parallel sliding

- Placement:

  - The motors are invisibly integrated into the pivot profile

### Water drainage:

- Downwards: One or more drainage openings of Ø50 mm are provided in the corner pieces or in the gutter. Each opening has a drainage trap that serves as a leaf catcher and forms the connection to a drain pipe (Ø50 mm) to be connected.

- Sidewards: A sideways-facing drainage pot in the corner pieces for drainage. It has a Ø50 mm to which a drain pipe (Ø50 mm) can be connected.

### Assembly:

All fixings (e.g. screws) are invisibly made of stainless steel.

Colour:

All visible aluminum profiles are powder coated in textured paint, RAL color of your choice (60–80 microns).

### Power supply and wiring:

Electrical cables are fully integrated, invisibly, into the terrace covering

## Technical data:

### Maximum weight of snow load:

Meets the static calculations according to DIN-EN regulations (Eurocode)

The load-bearing capacity depends on the dimensions and strength of the supporting structure.

With a maximum span of 4500mm, closed louvres have a load-bearing capacity of at least 520N/m².

### Maximum wind speed when controlling the blades:

120 km per hour (with closed blades)
160 km/h (with stacked blades)

### Wind resistant up to:

50 km per hour (with closed blades)

### Water drainage and precipitation rate:

The terrace covering can handle precipitation rate that corresponds to a rain shower with an intensity of 150 l/m2.h, which a maximum duration of 2 minutes. This type of rainfall occurs on average every 10 years. (Belgian rainfall statistics: NBN B 52-011)

## Warranty

- 10-year product warranty on the structure, subject to registration (all defects that may occur during normal household use and regular maintenance) (standard 5 years)

- 10-year warranty on the paint finish, color, and gloss of the aluminum profiles, subject to registration (standard 5 years)

- 5-year warranty on Somfy and Renson motors.

- 5-year warranty on Somfy controls and sensors.

- 5-year warranty on Lineo Luce, Lineo Fix and Protecto

- 2-year warranty on electrical components (heating, speakers, lighting, etc.)

- 2-year warranty on the rain sensor

## Options

### Up/Down LED lighting:

Integrated LED lighting along the full length of the inner side of the frame equipped with direct and/or indirect lighting

- Up

  - Warm white light, 120 LEDs/M | +/- 2800 K | 550-680 lumens/m

  - Pure white light, 120 LEDs/M | +/- 5000 K | 550-680 lumens/m

  - RGB 60 LEDs/m | 550-700 lumens/m

- Down

  - Warm white light, 120 LEDs/M | +/- 2800 K | 550-680 lumens/m

  - Pure white light, 120 LEDs/M | +/- 5000 K | 550-680 lumens/m

Beam heat & sound

### Beam:

  - The patio cover can be equipped with a Beam module. This is an aluminium housing that can be fitted with heating elements (Heat) and speakers (Sound).

  - Can be mounted on the inside of the frame profiles on the span side.

  - The module can be continuously adjusted with a 30° pivoting range.

### Heat:

- Heating element via radiant heat integrated in the Beam module.

- Heating element finished with flat black ceramic glass plate

- Power: 2400 W/Heat, Current = 12A/Heat, IP65

- RTS or IO control (same handheld transmitter as the louvres)

### Sound:

- 4 Ohms Marine Speakers IP65 , 2 speakers integrated in the Beam module

- 120W Peak Power , Frequency Response: 90Hz - 22kHz

- Cover plate in black anodised aluminium

- To be connected to an amplifier / radio via the supplied audio cables or with integrated amplifier for autonomous operation and control via smartphone or tablet (incl. bluetooth)

- Dimensions W x H: 180 x 180 mm

### Rain sensor kit:

- Rain sensor, fixing materials and accessories

- Closes the blades automatically when there is rain

- It is mounted on a slight incline and has a tiny heating power element for better sensor drying

### Wind, snow, frost:

- Adjustable via the app. Weather data is retrieved via the internet.

## Norms

This product is made to conform to, satisfies and/or has been tested according to the norm: EN 13561