# INTEGRATED CANOPY WITH FIXED ROOF

## Manufacturer

RENSON Outdoor nv, Polydore Rensonstraat 8, 9770 Kruisem – Belgium
Tel. +32(0)56 30 30 00, info@renson.be, www.renson-outdoor.com

(You can omit the text highlighted in red according to your choice)

## Description

Aero® Canvas is a horizontal aluminium canopy featuring an upper roof made of profiled steel sheeting and an aesthetic ceiling made of tightly tensioned fiberglass fabric, which is integrated into an existing structure.

## Dimensions

Span (longitudinal direction of the louvres):	min. 1200 mm – max. 4500 mm

expandable with an intermediate beam up to max. span 6000 mm
Pivot (lateral direction of the louvres):	min. 2615 mm – max. 6055 mm

For your information:

The span side corresponds to the width of the steel sheets.

The pivot side is the side along which the different steel sheets are placed next to each other.

## System implementation

### Frame:

- A sturdy surrounding frame made of extruded aluminium profiles

- Connection of adjacent frame profiles without corner caps

- Each frame profile is equipped with mounting brackets to fix the tensioned ceiling and the upper roof

- Height: 230 mm

- Pitch/Slope: 0°

### Upper roof:

- Profiled aluminium steel sheet, equipped on the underside with an anti‑condensation layer to prevent condensation droplets from falling onto the tensioned ceiling below

- The upper roof is fixed to the top frame via the mounting brackets

Ceiling:

- The fixed roof is finished on the underside with a horizontal fiberglass fabric, tensioned tightly using fabric-tensioning profiles

- The fabric is attached all around using zippers, which secure it into a fabric-tensioning profile and to the mounting brackets on the top frame

- The fabric-tensioning profiles can be pulled closer to the frame, thereby tensioning the ceiling fabric and giving it a clean, taut appearance

### Water drainage:

- The upper roof (steel sheet) is installed with a 20 mm slope. Water drains through a perforated profile on the side of the upper roof into the gutter, which is integrated into the profiles of the top frame and runs around the entire canopy.

- Downwards: One or more drainage openings of Ø50 mm are provided in the corner pieces or in the gutter. Each opening has a drainage trap that serves as a leaf catcher and forms the connection to a drain pipe (Ø50 mm) to be connected.

- Sidewards: A sideways-facing drain pot runs from the inside of the frame to the outside of the frame. It has a Ø50 mm drain pipe to which a drain pipe (Ø50 mm) can be connected.

### Assembly:

All fixings (e.g. screws) are invisibly made of stainless steel.

### Colour:

All visible aluminum profiles are powder coated in textured paint, RAL color of your choice (60–80 microns).

### Power supply and wiring:

Electrical cables are fully integrated, invisibly, into the terrace covering

## Technical data:

### Maximum weight of snow load:

### Wind resistant up to:

120 km per hour

### Water drainage and precipitation rate:

The terrace covering can handle precipitation rate that corresponds to a rain shower with an intensity of 120 l/m2.h, which a maximum duration of 2 minutes. This type of rainfall occurs on average every 10 years. (Belgian rainfall statistics: NBN B 52-011)

## Warranty

- 10-year product warranty on the structure, subject to registration (all defects that may occur during normal household use and regular maintenance) (standard 5 years)

- 10-year warranty on the paint finish, color, and gloss of the aluminum profiles, subject to registration (standard 5 years)

- 5-year warranty on Somfy and Renson motors.

- 5-year warranty on Somfy controls and sensors.

- 5-year warranty on Lineo Luce, Lineo Fix and Protecto

- 2-year warranty on electrical components (heating, speakers, lighting, etc.)

- 2-year warranty on the rain sensor

## Options

### Up/Down LED lighting:

Integrated LED lighting along the full length of the inner side of the frame equipped with direct and/or indirect lighting

- Up

  - Warm white light, 120 LEDs/M | +/- 2800 K | 550-680 lumens/m

  - Pure white light, 120 LEDs/M | +/- 5000 K | 550-680 lumens/m

  - RGB 60 LEDs/m | 550-700 lumens/m

- Down

  - Warm white light, 120 LEDs/M | +/- 2800 K | 550-680 lumens/m

  - Pure white light, 120 LEDs/M | +/- 5000 K | 550-680 lumens/m

## Norms

This product is made to conform to, satisfies and/or has been tested according to the norm: EN 13561