SURFACE MOUNTED PRESSURE-RELIEF LOUVRE TYPE RENSON 433S

SPECIFICATION SHEET

Louvre type Renson 433S is a surface mounted pressure-relief louvre and has the following properties:

- Aesthetics

  - Blade pitch = 37 mm

  - Invisible fixings

  - mitred, pressed corners

- Operation

Synchronously moving blades to eliminate the risk of rattling

- Airflow

  - Closed up to +/- 4 Pa pressure drop over the louvre

  - Dependent of dimensions and pressure drop:

| Dimensions (wxh) | Airflow at 10Pa pressure drop | Airflow at 25Pa pressure drop |
| --- | --- | --- |
| 173mm x 173mm | 15 m³/h | 142 m³/h |
| 210mm x 210mm | 37 m³/h | 166 m³/h |
| 246mm x 246mm | 50 m³/h | 361 m³/h |

- Material

  - extruded aluminium profiles (AlMgSi0.5, EN AW 6063 T66)

  - surface treatment:

    - anodised in natural colour EV6/EV1 (20 micron): pretreated and anodised OR

    - polyester powder coating in RAL colour according to the Qualicoat standard

- Louvre depth: 29mm