ENDURA® tWIST

Renson Ventilation, IZ 2 Vijverdam, Maalbeekstraat 10, 8790 Waregem – Belgium

Tel. 01622 754 123, vents@rensonuk.net www.renson.eu

DESCRIPTION  (red highlighted text can be deleted according to your choice)

- Type: decentralised window ventilator with heat recovery, continuous supply of fresh air and simultaneous extraction of used air.

- Installation: horizontally (above window profile) or vertically (perfectly combinable with screens/roller shutters)

- Installation without ducts, so ideal solution for renovation

- With regenerative alternating technology:

  - continuous supply of fresh air and simultaneous extraction of polluted air: two ventilation units alternate cyclically between supply and extraction of air

  - frost- and condensation- free

- Modular system: 2 ventilation units per product, with 1 up to 6 fans per ventilation unit

(more fans = more airflow)

- Thermally broken

- Low sound generation

- Low power consumption

- Bypass function: no pre-heating of the supplied air during summer

- Natural ventilation-function: natural instead of mechanical supply and extraction

- Silent mode-function: limited power consumption and sound generation (e.g. for night time)

- Fully closable: fans turn 90° until closed position

- Material and surface finishing:

  - aluminium interior and exterior profiles

  - available powder coated in any RAL colour

- Various air supply possibilities

  - downwards

  - upwards

- Watertight rain cap

- Control options:

  - optional: Touch Display with integrated IAQ-sensors

  - optional: Integration in Building Management System (BMS) possible

  - optional: Button control

- Optionally available with filters (G3 or F7)

- Optionally available with demand controlled: Indoor Air Quality (IAQ)

- Ease of maintenance: ventilation units and filters easily accessible by opening the interior profile

- European EcoDesign label: A / A+

Technical specifications

| Endura Twist* |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of fans |  | 2x1 | 2x2 |  | 2x3 |  | 2x4 | 2x5 |  |  | 2x6 |
| No filter | Qmin (m³/h) | 6.8 m³/h | 13.5 m³/h |  | 20.3 m³/h |  | 27 m³/h | 33.8 m³/h |  |  | 40.5 m³/h |
|  | Qnom (m³/h) | 13.5 m³/h | 27 m³/h |  | 40.5 m³/h |  | 54 m³/h | 67.5 m³/h |  |  | 81 m³/h |
|  | Qmax (m³/h) | 27 m³/h | 54 m³/h |  | 81 m³/h |  | 108 m³/h | 135 m³/h |  |  | 162 m³/h |
| Coarse 45% | Qmin (m³/h) | 6 m³/h | 12 m³/h |  | 18 m³/h |  | 24 m³/h | 30 m³/h |  |  | 36 m³/h |
|  | Qnom (m³/h) | 12 m³/h | 24 m³/h |  | 36 m³/h |  | 48 m³/h | 60 m³/h |  |  | 72 m³/h |
|  | Qmax (m³/h) | 24 m³/h | 48 m³/h |  | 72 m³/h |  | 96 m³/h | 120 m³/h |  |  | 144 m³/h |
| ePM1 75% | Qmin (m³/h) | 4.9 m³/h | 9.8 m³/h |  | 14.7 m³/h |  | 19.6 m³/h | 24.5 m³/h |  |  | 29.4 m³/h |
|  | Qnom (m³/h) | 9.8 m³/h | 19.6 m³/h |  | 29.4 m³/h |  | 39.2 m³/h | 49 m³/h |  |  | 58.9 m³/h |
|  | Qmax (m³/h) | 19.6 m³/h | 39.2 m³/h |  | 58.8 m³/h |  | 78.4 m³/h | 98 m³/h |  |  | 117.7 m³/h |
| Thermical efficiency level (EN13141-8) |  | max. 80% |  |  |  |  |  |  |  |  |  |
| Sound reduction (ISO 10140-2) Dn,e,w (C;Ctr) in open position |  | max. 40 (0;-3) dB |  |  |  |  |  |  |  |  |  |
| Sound reduction (ISO 10140-2) Dn,e,w (C;Ctr) in closed position |  | max. 50.6 (-1;-4) dB |  |  |  |  |  |  |  |  |  |
| Sound generation (ISO 3741:2010) Lp: measured at 2m distance from the window vent |  |  |  |  |  |  |  |  |  |  |  |
| No filter | Qmin | 18.1 dB(A) | 21.1 dB(A) |  | 22.8 dB(A) |  | 24.1 dB(A) | 25 dB(A) |  |  | 25.8 dB(A) |
|  | Qnom | 34.3 dB(A) | 37.3 dB(A) |  | 39.1 dB(A) |  | 40.3 dB(A) | 41.3 dB(A) |  |  | 42.1 dB(A) |
|  | Qmax | 40 dB(A) | 43 dB(A) |  | 44.8 dB(A) |  | 46 dB(A) | 47 dB(A) |  |  | 47.8 dB(A) |
| Coarse 45% | Qmin | 18 dB(A) | 21 dB(A) |  | 22.8 dB(A) |  | 24 dB(A) | 25 dB(A) |  |  | 25.8 dB(A) |
|  | Qnom | 34.2 dB(A) | 37.2 dB(A) |  | 39 dB(A) |  | 40.2 dB(A) | 41.2 dB(A) |  |  | 42 dB(A) |
|  | Qmax | 39.9 dB(A) | 42.9 dB(A) |  | 44.7 dB(A) |  | 45.9 dB(A) | 46.9 dB(A) |  |  | 47.7 dB(A) |
| ePM1 75% | Qmin | 19.6 dB(A) | 22.6 dB(A) |  | 24.4 dB(A) |  | 25.6 dB(A) | 26.6 dB(A) |  |  | 27.4 dB(A) |
|  | Qnom | 36.7 dB(A) | 39.7 dB(A) |  | 41.5 dB(A) |  | 42.7 dB(A) | 43.7 dB(A) |  |  | 44.5 dB(A) |
|  | Qmax | 42.7 dB(A) | 45.7 dB(A) |  | 47.5 dB(A) |  | 48.7 dB(A) | 49.7 dB(A) |  |  | 50.5 dB(A) |
| Power Consumption |  |  |  |  |  |  |  |  |  |  |  |
| No filter | Qmin | 2.5 W | 3.4 W |  | 4.3 W |  | 5.2 W | 6.1 W |  |  | 7 W |
|  | Qnom | 3.1 W | 4.6 W |  | 6.1 W |  | 7.6 W | 9.1 W |  |  | 10.6 W |
|  | Qmax | 5.2 W | 8.8 W |  | 12.4 W |  | 16 W | 19.6 W |  |  | 23.2 W |
| Coarse 45% | Qmin | 2.7 W | 3.8 W |  | 4.8 W |  | 5.9 W | 6.9 W |  |  | 8 W |
|  | Qnom | 3.6 W | 5.5 W |  | 7.4 W |  | 9.4 W | 11.3 W |  |  | 13.2 W |
|  | Qmax | 6 W | 10.4 W |  | 14.7 W |  | 19.1 W | 23.4 W |  |  | 27.7 W |
| ePM1 75% | Qmin | 2.8 W | 3.9 W |  | 5.1 W |  | 6.2 W | 7,3 W |  |  | 8.4 W |
|  | Qnom | 3.7 W | 5.7 W |  | 7.8 W |  | 9.8 W | 11.8 W |  |  | 13.9 W |
|  | Qmax | 6.2 W | 10.8 W |  | 15.3 W |  | 19.9 W | 24.4 W |  |  | 29 W |
| U-value (EN ISO 10077-2) |  | 1.0 W/m²K |  |  |  |  |  |  |  |  |  |
| Watertightness (in open position) (EN 13141-1:2004) |  | up to 150Pa |  |  |  |  |  |  |  |  |  |
| Fall protection (DIN EN 13049) |  | Class 5 (if correctly mounted) |  |  |  |  |  |  |  |  |  |
| Height |  | 110 mm |  |  |  |  |  |  |  |  |  |
| Min. length |  | 750 mm |  | 1000 mm |  | 1250 mm | 1500 mm |  | 1750 mm | 2000 mm |  |
| Max. length |  | 6000 mm |  |  |  |  |  |  |  |  |  |
| Depth |  | 320 mm (345 mm with design exterior cover) |  |  |  |  |  |  |  |  |  |
| Filter |  | optional G3 or F7 filter |  |  |  |  |  |  |  |  |  |
| Device class |  | I |  |  |  |  |  |  |  |  |  |
| Usable temperature-range |  | from ‒15 °C to +45 °C |  |  |  |  |  |  |  |  |  |
| Resistant to frost |  | √ |  |  |  |  |  |  |  |  |  |
| Condensation free |  | √ |  |  |  |  |  |  |  |  |  |
| Voltage |  | 230V / 50Hz |  |  |  |  |  |  |  |  |  |
| Voltage of device (internal) |  | 15V DC |  |  |  |  |  |  |  |  |  |
| Supply voltage |  | 230 V AC ± 10% |  |  |  |  |  |  |  |  |  |

DIMENSIONS

- Height:						110 mm

- Depth:						320 mm (345 mm incl. design exterior cover)

- Length: 						2x1 fans: 750 mm – 6000 mm

2x2 fans: 1000 mm – 6000 mm

2x3 fans: 1250 mm – 6000 mm

2x4 fans: 1500 mm – 6000 mm

2x5 fans: 1750 mm – 6000 mm

2x6 fans: 2000 mm – 6000 mm

*  More information: see Technical data sheet.

Technical specifications based upon internal test results. Possible variations in external testing.