SPECIFICATION SHEET (Belgium): HEALTHBOX 3.0 SMARTZONE Version 09/01/2024 Renson N.V.

_____________________________________________________________________________________________

Healthbox 3.0 SmartZone has been developed for controlled central extraction of contaminated air in a home/residential unit/apartment, etc. and forms an integral part of the energy-saving C+ ventilation system.

The C+ ventilation system is guaranteed to function optimally when the following three elements are present and coordinated:

Supply air		: In dry rooms via self-regulating RENSON® window vents (class P3 or P4)

Transfer air		: 25 m³/h at 2 Pa – 50 m³/h at 2 Pa (to kitchen)

Extraction air	: Healthbox 3.0	: Autonomous demand-driven home ventilation unit in conformity with

NBN EN 13141-6 individual detection and extraction in each room are

regulated by control valves mounted externally on the motor unit

and controlled and supplied directly from the motor unit.

Air ducts	: Easyflex – best airtightness class: D – material: PE

Exhaust	: RENSON® roof/wall exhaust with low pressure loss

# EPB RESIDENTIAL

- Healthbox 3.0 is included in the energy performance and indoor climate (EPB) product details database

- Fan and ventilation group

- Demand-driven ventilation systems (EPW)

- Parameters for EPB software

- System				: 3 variant configs

- System C+ Healthbox 3.0 SmartZone 0.43

Mechanical extraction in all humid rooms and dry rooms. In an open kitchen, the CO2-driven extraction in the kitchen will suffice and no additional separate extraction must be fitted in the living room! Local control and detection of all mechanical extraction flow rates.

- System C+ Healthbox 3.0 SmartZone 0.50

Mechanical extraction in all humid rooms and bedrooms. Local control and detection of all mechanical extraction flow rates!

- System C+ Healthbox 3.0 SmartZone 0.61

Mechanical extraction in all humid rooms and bedrooms. Local control and detection in all humid rooms, central control and detection in two or more bedrooms.

- Control strategy			: Speed control and variable pressure

- Type of speed control		: EC motor with commutation control

- Max. power			: According to in situ measurement (with Pelec, fan, max. = max. 85W)

- Demand-driven control by-pass	: Yes (Breeze)

- Demand-driven control reduction factor:

freduc, vent, heat:           according the config: 0.43 or 0.50 or 0.61

freduc, vent, cool:           1.00

freduc, vent, overheat:      1.00

# SPECIFICATIONS

- Airflow

- 475 m³/h (135 Pa)

- 430 m³/h (200 Pa)

- Maximum power fan unit

- Max. 35W (225 m³/h)

- Max. 80W (400 m³/h)

- Noise power level (cf. Ecodesign)

- Max. 34 dB(A) (225 m³/h)

- Max. 47 dB(A) (475 m³/h)

- Dimensions (L x W x H)

- Without control valves: 390 x 443 x 200 mm

- With control valves: 567 x 567 x 200 mm

# PRODUCT FEATURES

- Control valves with electronic sensors

- Measures air quality in the extracted airflow per room 24/7 and regulates the airflow accordingly:

- Absolute CO2 detection: proportional valve control depending on the set CO2 level

- Absolute & dynamic H2O moisture detection: proportional and/or open/closed valve control

- Dynamic VOC odour detection: open/closed valve control

- Type of control per room:

Kitchen (CO2+H2O) – Laundry room / Bathroom (H2O) – Bathroom+Toilet (H2O+VOC) – Toilet (VOC) – Bedroom (CO2)

- Elliptical damper blade for low-noise operation

- Connected externally to the motor unit

- Valve collectors

  - Centralised and/or decentralised connection (via RJ45 patch cable)

  - Connect up to three control valves to one motor unit supply point

  - Up to max. 11 control valves can be connected (using 2 valve collectors)

- Automatic calibration (commissioning)

  - Guarantee/control of the attainment of the extraction airflows

- Fan

  - EC motor made of galvanised steel

  - Impeller ø180 for extremely quiet and energy-efficient operation

- Fan with active variable pressure control

Fan speed is continuously controlled to achieve the extraction required for each room at the lowest possible pressure level (the valve blade of one control valve is always fully open)

- Automatic Breeze function

The fan will temporarily (automatically) ventilate at nominal airflow rate if coolness is required

AVAILABLE DIGITAL INTERACTION

- Directly, via integrated Ethernet connection or via Wi-Fi dongle

- Automatic software update if Healthbox 3.0 is online

- Digital communication:

- Communication with resident: via resident app and web portal

- Communication with smart home/home automation: via API and/or via switching module

- Resident app: visual representation of real time/historic air quality and ventilation level in the home, down to room level. Colours indicate the air quality per home/room. Ventilation profile configuration.

# CONTROLS

Optional (temporary) manual adjustment of the ventilation extraction rate is possible via the controls.

- Optional potential-free multi-position switch – wired

- Resident app available: airflow rate per room adjustable for a certain time period - control of Healthbox 3.0 inside and outside the local network

# COMBINABLE PRODUCTS

- Flat ALU extraction grilles without a control valve as an extraction point in the room

- Easyflex air ducts: ducts for air transport, best airtightness class: D

- Invisivent® (COMFORT) window ventilation

- Non-motorised extractor hood connected to the Healthbox 3.0 to extract cooking vapours at up to 300 m³/h (application in apartment buildings)