RENSON WINDOW VENTILATION® SPECIFICATIONS

Brand	: Sliding window ventilators

Type 		: THL100

Manufacturer and reference:

The THL100 is manufactured and supplied by Renson Ventilation Tel: +32 (0)56 62 71 11

Description:

The RENSON® THL100 is a thermally broken, horizontal sliding ventilator for installation on glass (or at transom). It has louvres at the outside, and a slider at the inside. Not suitable for installation in coastal environment or near the beach.

System operation and performance:

Not self-regulating

Thermal conductivity: U-value = 3.9 W/m2K

Watertightness: up to 400Pa in closed position

Watertightness in open position: n.p.d.

Air leakage at 50Pa : n.p.d.

Airflow and Sound Reduction:

Dn,e,w (C;Ctr) open (dB)		22 (0;-1)

Dn,e,w (C;Ctr) closed (dB)		42 (-1;-2)

Q at 1 Pa (l/s/m	)		13,2

Q at 1 Pa (m³/h/m)		47,4

Q at 2 Pa (l/s/m	)		18,7

Q at 2 Pa (m³/h/m)		67,2

Q at 10 Pa (l/s/m)		41,6

Q at 20 Pa (l/s/m)		58,9

Equivalent area	(mm²/m)	16759

Dimensions / Size Range:

Glass reduction: 129 mm

Height: 144 mm

Fits to glass depths of: 15, 20, 24 or 28 mm (15 mm not available for installation at transom)

Maximum length: 3500 mm

Ventilator Construction:

Profile: manufactured from aluminium alloy Al Mg Si 0.5

Surface treatment:

Satin anodised (EV6/EV1) (20 micron): pre-treated and anodised.

Polyester powder coating in any RAL colour (60 to 70 micron): aluminium profiles pre-treated to resist corrosion to guarantee (dual colour possible).

End cap colours: black.

Controls Option:

Manual (continuous adjustment)

Rod, chain or cord are optional (continuous adjustment)

Motorised

Installation:

Horizontal, glazed-in installation. Also installation at transom possible.