## Product features

### Supports and clips:

### Supports can be fixed horizontally with Renson's direct fixing system (patented) or on a perfectly aligned underlying supporting structure.

- Supports can be bent for cladding of a curved wall (smallest radius 1000 mm).

- Plastic clips (patented) mount onto the horizontal supports without screws.

- Plastic clips allow thermal expansion without creaking noises.

### Basic profiles:

- Individual vertical profiles, available in various shapes (block or rib-shaped)

- The individual vertical profiles combine with each other allowing creation of personalized wall cladding (MIX)

- The individual vertical profiles allow a variety of colour combinations

- Each basic profile can be fixed in the nominal position or can be extended in or out by 2 mm, allowing the width of the façade to be covered in a flexible manner, with a maximum extension or contraction of 32 mm/rm

- The system is vandalism-proof, but the profiles can still be removed individually if necessary, without having to disassemble the rest of the wall

- The profiles can be rotated in any direction, no matter which position they are in

| Profile type | Shape | Maximum Length (mm) | Width (mm): | Free Depth (mm) | System Depth (profile + support) (mm) | Weight incl. support (kg/m2) |
| --- | --- | --- | --- | --- | --- | --- |
| EVEN |  | 6.000 | 60 | 25 | 61 | 13 |
| BLOCK25 |  | 6.000 | 60 | 25 | 61 | 12 |
| BLOCK16 |  | 6.000 | 60 | 16 | 52 | 10 |
| BLOCK16L CONE |  | 6.000 6.000 | 120 60 | 16 25 | 52 61 | - 11 |

BLOCK16L only applicable in MIX

### Angle profiles and interchangeable profiles:

- The angle profiles (indoor & outdoor) ensure seamless corner connections.

- Exterior angle:  BLOCK16, BLOCK25, EVEN

- Interior angle: BLOCK25 and EVEN

- With the interchangeable profiles, it is possible to change direction in order to integrate perfectly  doors, windows or gates into a form-fitting unit in the façade.

### Fitting profiles:

- More clearance for use with doors or gates

- Can be provided with a rubber seal

## Material and surface treatment profiles

- Made of extruded aluminium Al Mg Si 0.5

- Supports technically anodized 5μ.

- The aluminium profiles are finished with a polyester powder-coating in RAL architectural textured coating (60 to 80 microns) in accordance with the Qualicoat standard.

- The profiles with a wooden design first receive a powder coating primer layer in accordance with the Qualicoat standard, upon which the finishing is applied using a plasticizer-free wood-look PVC film with a transparent PVDF protective top layer.

## Options

### Direct fixing:

- Durable and fireproof solution. Fast mounting of the supports directly on the wall with anchor plug (patented).

### Profiles for window finish:

- To finish the edges next to and above the window, with invisible fastening, in the same surface treatment and colour of the Linarte wall.

Integration of gates and doors:

- Finishing profiles for on and next to gates and doors

- Mounting profile for mounting an optical sensor type Cedes GridScan/Mini (excl.) for sectional doors

- Lowered clips for mounting the Linarte profiles on a tilting door (maximum system depth for BLOCK 25: 41 mm)

Semi-open window cladding:

- The profiles that run in front of the window are aesthetically finished at the rear with a cover profile.

The profiles EVEN and BLOCK25 can be personalized by adding LED or wood inserts. 
These can also be applied at a later stage.

### LED-modules:

- 24V, IP67-potted profile without diffuser. Available in lengths of 267, 455, 830 or 1205 mm. Including connectors. Warm white Light +/- 3500K. Dimmable.

### Indirect LED lighting:

- 24V, IP66 LED strip. Available in lengths up to 6200 mm. To be mounted at the bottom of the Linarte support profile. Warm white light +/- 3000K.

### Wooden inserts:

- Thermo-es, available in lengths of 267, 455, 830 or 1205 mm. Can be mounted with a fixed or removable clip system.

### House numbers:

- Numbers from 0 to 9, indices from A to D in wood fillings (thermo-es). Applicable in the EVEN-profiles, height +/- 420 mm.

### Integration features:

- The mailbox, intercom, electrical outlet and outdoor tap all integrate seamlessly into the whole.

### Fire clips:

- Each Linarte profile can be secured with a fire clip if required.

## Assembly

Can be used as a ventilated facade system if the Renson direct fixing system is used, or if the Linarte facade cladding is fixed on a firm and flat substructure that has been professionally installed and complies with local regulations. Take account of sufficient cavity thickness.

Can be used as a, when mounted on a substructure with wooden (CLS/SLS) or aluminium supports, according to the processing guidelines of the manufacturer.

## Standards

- Aluminium alloy: Al Mg Si 0,5 (F25)

  - Standardization: EN AW-6063

  - Hardening: T66

- Aluminium pre-treatment:

  - Standard DIN 50021 SS

- Strength calculations are based on the following standards:

  - EN 1999-1-1 : Calculation of structures in aluminium

  - NBN B-03-002-2 : Wind load - Dynamic effects

  - EN 1991-1-4 : Wind Load

- Fire resistance: B-s1, d0

  - Classification of reaction to fire performance in accordance to EN 13501-1: 2013