# TERRACE COVERING WITH ROTATABLE AND RETRACTABLE ALUMINIUM BLADES

## Manufacturer

RENSON Outdoor nv, Polydore Rensonstraat 8, 9770 Kruisem – Belgium
Tel. + 32 (0)56 30 30 00, info@renson.be, www.renson-outdoor.com

(You can omit the text highlighted in red according to your choice)

## Description

Camargue Skye® is an aluminium terrace covering comprising a horizontal sun protection roof, equipped with a rotatable and retractable extruded aluminium blade system, which is supported by one, two or four vertical columns, depending on the installation type. The terrace covering can be installed as a free-standing unit or against a façade.

## Dimensions

Span:	Min. 1800 mm

Max. 4500 mm

Pivot:	Min. 3250 mm

Max. 6200 mm

Free passage height 	Maximum 2800 mm

Total height, without the motor box:	Free passage height + 260 mm

Total height with blades in the vertical position: free passage height + 260 mm + 95 mm

Information:
The "Span side" corresponds to the span of the blades
The "Pivot side" is the side where the blades rotate (or pivot)

## System implementation

### Frame:

- A sturdy surrounding frame made of extruded aluminium profiles

- Height: 	260 mm

- Depth of the Span frame profile: 150 mm

- Depth of the Pivot frame profile: 330 mm

- Inclination: 	0°

### Columns:

- The installation uses 1, 2 or 4 columns, depending on the installation type.

- Dimensions: 150 x 150 mm

- Consists of a central, cross-shaped profile, finished with L-profile s that can be implemented according to custom design, using them as side channels, column lights or as limitation of rapid passage

### Blades:

- Sun protection blades made of extruded aluminium

- Double-walled for handling a heavy snow load

- Rotatable through 135°

- Underside: completely flat

- Upper side: sharply defined edges + water drainage gutter

- Side edges equipped with hidden fixed L-profile to stop water run-back

- The specific design ensures water drainage when the blades are closed

- And water drains off when you rotate the blades after a shower

- Blade mounting:

  - Fitting into the pivot -sides using the stainless steel shafts

  - A slight inclination on the blades for optimum water drainage

### Mounting bases:

There are 3 types:

- Visible mounting bases with a visible ground plate

- Invisible mounting bases

- Cast mounting bases

### Motorisation:

- 4 in-line 24 V DC motors with planetary transmission

- RTS/IO

- How it works

  - Rotation: Worm and worm gear

  - Retraction/opening: Synchronised spindles with a trolley and chain system
Even and guaranteed parallel sliding

- Placement:

  - The motors are invisibly integrated into the pivot profile

### Water drainage:

- Integrated water drainage: Water from the slats and the invisible integrated gutters is drained through each column towards one Span side.

- Gutters are integrated into the columns on the Span side.

- If the mounting foot is sealed, the gutter can be connected to a water tank.

### Colour:

All visible aluminium profiles are coated in your choice of architectural textured RAL colour (60-80 microns)

### Assembly:

All fixings (e.g. screws) are made of stainless steel

### Power supply and wiring:

Electrical cables are integrated fully and invisibly into the terrace covering

## Technical data:

### Maximum weight of snow load:

See the chart below, according to the dimensions

### Maximum wind speed when controlling the blades:

50 km per hour

### Wind resistant up to:

120 km per hour (with closed blades)

### Water drainage and precipitation rate:

The terrace covering can handle precipitation rate that corresponds to a rain shower with an intensity 150 l/m2.h, which a maximum duration of 2 minutes. This type of rainfall occurs on average once every 10 years. (See the Belgian rain statistics: NBN B 52-011)

## Warranty

- Guarantee product for 10 years based on the structure (this is due to the defects during the normal use and regular use)

- 10-year warranty on the paint finish, color, and gloss of the aluminum profiles, subject to registration (standard 5 years)

- 5 years warranty on Somfy and Renson motors.

- 5 years warranty on Somfy controls and sensors.

- 2 years warranty on electrical components (heating, speakers, lighting, etc.).

- 2 years warranty on the rain sensor.

- 5 years warranty on the infills (loggias, glass sliding panels, fixed panels, exterior curtains).

- 5 years warranty on the Fixscreen technology: zip stays in the side channel + optimal attachment of the zip.

- 5 years warranty on fibreglass, polyester and acrylic fabrics.

## Options

### Led Lighting:

Colomno LED (no backorder possible) - Integrated LED lighting in the columns.

- Warm white light: 550 – 680 lm/m (120 LEDs/m)

- Pure white light: 550 – 680 lm/m (120 LEDs/m)

UpDown LED - Integrated LED lighting on the inside of the frame.
The Up/Down LED features upward and/or downward lighting providing indirect or direct ambient lighting.

- Up:

  - Warm wit licht 120 LEDs/M | +/-2800K | 550-680 lumen/m

  - Puur wit licht 120 LEDs/m | +/- 5000K | 550-680 lumen/m

  - RGB  60 LEDs/m | 550-700 lumen/m

- Down:

  - Warm wit licht 120 LEDs/M | +/-2800K | 550-680 lumen/m

  - Puur wit licht 120 LEDs/m | +/- 5000K | 550-680 lumen/m

### Loggia sliding panels

- Loggia sliding panels comprise 40 x 40, 40 x 55 or 40 x 70 mm powder coated aluminium frames, depending on the passage height. The 40 mm side is always facing towards the front

- This frame is filled in with:

  - Square, aluminium, fixed blades

  - Square, aluminium, rotatable blades

  - Square, Western Cedar, fixed blades

  - Square, Western Cedar, rotatable blades

  - Glass fibre screen fabric

- The sliding panels are suspended in aluminium upper rails and slide in an aluminium bottom rail

- Operation is manual

- The system can handle height differences of up to 5 cm, using a flexible lower guide system

- See the specification text for Loggia sliding panels for a detailed product description

### Integrated Fixscreens

- It is possible to integrate vertical sun protection fabric screens in the terrace coverings with a passage height ≤ 2.8 m

- The frame profiles of the basic structure also form the box for the overhead sun protection

- The side channels are made of extruded aluminium They are 3-piece

- The stepped shape in both components allows you to screw the side channels easily and in the proper manner onto the columns

- See the specification text for Fixscreens for a detailed product description

### Glass wall

- You can build glass sliding panels into terrace coverings with up to 2600 mm passage height, subject to the use of reinforced mounting bases on the adjacent columns

- The glass profiles are made of powder-coated extruded aluminium

- The control is manual

- The glass sliding panels are supported at the bottom by an extruded aluminium rail that is finished with a corrosion resistant guide channel

- The glass is 10 mm thick safety-glass

- You can close the glass wall securely using a lock that you can open and close with a key from either side.

### Beam Heat & Sound

Only possible on span, not on pivot

### Beam:

  - The patio cover can be equipped with a Beam module. This is an aluminium housing that can be fitted with heating elements (Heat) and speakers (Sound).

  - Can be mounted on the inside of the frame profiles on the span side.

  - The module can be continuously adjusted with a 30° pivoting range.

### Heat:

- Heating element via radiant heat integrated in the Beam module.

- Heating element finished with flat black ceramic glass plate

- Power: 2400 W/Heat, Current = 12A/Heat, IP65

- RTS or IO control (same handheld transmitter as the louvres)

### Sound:

- 4 Ohms Marine Speakers IP65 , 2 speakers integrated in the Beam module

- 120W Peak Power , Frequency Response: 90Hz - 22kHz

- Cover plate in black anodised aluminium

- To be connected to an amplifier / radio via the supplied audio cables or with integrated amplifier for autonomous operation and control via smartphone or tablet (incl. bluetooth)

- Dimensions W x H: 180 x 180 mm

### Rain sensor kit:

- Capacity: Rain sensor, mounting materials and accessories

- Action:

  - Closes the blades automatically when there is rain

  - Rotates the blades to their pre-set snow position whenever there is precipitation combined with a freezing temperature

- It is mounted on a slight incline and has a tiny heating element for better sensor drying

Wind, snow, frost:

Adjustable via the app. Weather data is retrieved via the internet.

## Norms

This product is made to conform to, satisfies and/or has been tested according to the norm: EN 13561