RENSON WINDOW VENTILATION® SPECIFICATIONS

Brand	: THM90evo

Type 		: THM90evo - THM90PBevo - THM90TRevo

Manufacturer and reference:

The THM90evo is manufactured and supplied by Renson Ventilation Tel: +32 (0)56 62 71 11

Description:

The RENSON® THM90evo is a thermally broken, self-regulating window ventilator for installation on the glass, under the glass, or at transom. The entirely flat perforated (series of 2.7 x 18.5mm) inner profile acts as an insect mesh, and makes this the perfect solution for integration in sliding doors.

System operation and performance:

Self-regulating

Thermal conductivity: U-value = 3.8 W/m2K

Watertightness: up to 650Pa in closed position

Watertightness: up to 100Pa in open position

Air leakage at 50Pa : <15% (in closed position)

Airflow and Sound Reduction:

Dn,e,w (C;Ctr) open (dB)		26 (0;0)

Dn,e,w (C;Ctr) closed (dB)		45 (-1;-1)

Q at 1 Pa (l/s/m	)		9,3

Q at 1 Pa (m³/h/m)		33,5

Q at 2 Pa (l/s/m	)		13,9

Q at 2 Pa (m³/h/m)		50,0

Q at 10 Pa (l/s/m)		13,5

Q at 20 Pa (l/s/m)		15,1

Equivalent area	(mm²/m)	11841

Dimensions / Size Range:

Glass reduction: 90mm

Height: 105mm

Fits to glass depths of: 	-     THM90evo : 20, 24, 28 or 33 mm

- THM90PBevo : 20, 24 or 28 mm

- THM90TRevo : 20 or 24 mm

Maximum length: 2500mm

Ventilator Construction:

Profile: manufactured from aluminium alloy Al Mg Si 0.5

Thermal break: PVC

Pivoting and self-regulating flap: PVC

End caps: ASA polymer type Luran S (colour-fast, weather and UV-resistant)

Surface treatment:

Satin anodised (EV6/EV1) (20 micron): pre-treated and anodised.

Polyester powder coating in any RAL colour (60 to 70 micron): aluminium profiles pre-treated to resist corrosion to guarantee (dual colour possible).

End cap colours: white or black (other colours available on request).

Controls Option:

Manual: end caps control (5 stepped positions)

Rod (standard, with reduction gear or sliding knob) or cord are optional (5 stepped positions)

Motorised: 24V DC (only for made to measure vents)

Installation:

Glazed-in installation (on timber, aluminium or PVC window frames). The THM90evo can be installed on the glass, at the bottom of the window (THM90PBevo , 100kg max) or at transom (THM90TRevo). The THM90evo should only be used for installation on ground floors.

Standards:

EN ISO 140-10, EN ISO 717-1, EN 1026, EN 1027, EN 13141-1, EN 12020-2, EN AW 6063 T66, NBN D50-001, EN 1077-2, DIN16491.