RENSON WINDOW VENTILATION® SPECIFICATIONS

Brand	: Sonovent®

Type 		: Sonovent® Compact

Manufacturer and reference:

The Sonovent® Compact is manufactured and supplied by Renson Ventilation

Tel: 01622 754 123 Email: vents@rensonuk.net

Description:

The RENSON® Sonovent® Compact is a thermally broken, self-regulating and acoustic window ventilator. The perforated inner profile acts as an insect mesh (removable for cleaning purposes).

Optionally available with Pollux filter: for strongly fine dust and pollen impacted environments.

System operation and performance:

Self-regulating

Thermal conductivity: U-value = 3.45 W/m2K

Watertightness up to 150Pa in open position

Watertightness up to 650Pa in closed position

Air leakage at 50Pa: <15% in closed position

Airflow and sound reduction:	airslot 10 mm		airslot 13mm		airslot 15 mm

Dn,e,w (C;Ctr) open (dB)			36 (0;-1) 		35 (0;-1) 		33 (0;-1)

Dn,e,w (C;Ctr) closed (dB)			n.p.d.			n.p.d.			n.p.d.

Q at 1 Pa (l/s/m	)			12,1			15,2			19,4

Q at 1 Pa (m³/h/m)			43,4			54,5			69,8

Q at 2 Pa (l/s/m	)			16,4			18,8			19,9

Q at 2 Pa (m³/h/m)			58,9			67,7			71,5

Q at 10 Pa (l/s/m)			17,3			18,8			18,6

Q at 20 Pa (l/s/m)			17,8		    	18,7			19,1

Equivalent area (mm²/m)		15334		   	19278			24687

Dimensions / Size Range:

Glass reduction: 78 mm

Height: 75 mm (box height) / 95 mm (total height with flanges)

Fits to glass depths of: 20, 24, 28, 32 or 36

Maximum length: 2000 mm (glazed-in) or 2500 mm (at transom)

Ventilator Construction:

Profile: manufactured from aluminium alloy Al Mg Si 0.5.

Thermal bridge: extruded uPVC.

Pivoting and self-regulating flap: PVC.

End caps: ASA polymer type Luran S (colour-fast, weather and UV-resistant).

Sound absorbing material: non-combustible mineral wool.

Surface treatment:

Polyester powder coating in any RAL colour (60 to 70 micron): aluminium profiles pre-treated to resist corrosion to guarantee (dual colour possible)

End cap colours: white or black (other colours available on request)

Controls Option:

Manual, rod or cord optional (continuous adjustment)

Motorised: 24V DC (continuous adjustment)

Installation:

Can be installed glazed-in (timber, PVC or aluminium) or at transom (using additional transom profiles).

Standards:

EN ISO 140-10, EN ISO 717-1, EN 1026, EN 1027, EN 13141-1, EN 12020-2, EN AW 6063 T66, NBN D50-001, EN 1077-2.